ADDRESS and PREMISE

IDENTIFICATION

(Based on 2012 edition of International Fire Code as amended)

Location of Address Numbers

The capability of an emergency response crew to quickly find the correct address of a building or residence in an emergency is very critical. Time lost trying to find an address may be time lost trying to save a life.

New and existing buildings shall have approved address numbers, building numbers or approved building identification placed in a position that is plainly legible and visible from the street or road fronting the property.

Size of Numbers

Address numbers shall be Arabic numerals or alphabet letters. Numbers shall be a minimum of four (4) inches high with a minimum stroke of width of one-half (0.5) inch. Larger numbers are recommended.

These numbers shall contrast with their background. All residential house numbers shall be located on the portion of the residence which projects nearest to the street or road.

Addresses of Remote Buildings

In cases where the structure is built far enough from the roadway that it is impossible to place numbers that are legible from the roadway, a sign post placed at the intersection of the roadway and driveway denoting the address may be required by the Fire Chief.

Where buildings are not visible from the street, or more than one building address is on a site, or the building is located more than 100 feet from the street, (i.e., buildings on a long driveway or private roadway, an apartment complex with

multiple addresses), the address numbers shall also be posted in a permanent manner at the

intersection of the private driveway or easement ad the public street.

If more than one building is located in this remote area, the addressing shall indicate the address range of the buildings accessible from the private driveway or easement.

Addressing of Rear Doors

The rear entrance or access doors of all malls, strip centers, commercial center buildings, or other areas with multi-tenant spaces shall be identified with the appropriate address number and business name.

The address numbers shall be at least four (4) inches high and three-quarters (3/4) inch stroke.

The Fire Department may require the installation of address numbers/letters on other locations to prevent confusion in the event of an emergency.

Addressing Individual Units within a

Mall or Shopping Center

Any area occupied by tenants of a mall or shopping center shall have the space identified by numbers that are a minimum of four (4) inches high and no less than a one-half (1/2) inch stroke. The numbers shall be plainly visible from a distance of at least 50 feet from the entrance.

Hotel, Motel Rooms and Apartments

All exterior access doors of individual dwelling units located within apartment buildings, hotels and motels shall be identified by unit numbers/letters at least two (2) inches high and no less than one-quarter (1/4) inch stroke.

Office Suites

Office Suites located within commercial buildings shall be identified with letters/numbers a minimum of two (2) inches high and no less than a one-quarter (1/4) stroke.